MOTOROLA

FREEWARE

8-BIT CROSS ASSEMBLERS

USER'S MANUAL

EDITED BY

KEVIN ANDERSON

FIELD APPLICATIONS ENGINEER

MODIFIED FOR REV 1.02

10/27/1989

BY

Bruce L. Olney

TABLE OF CONTENTS

CHAPTER 1.........................................................  1

1.1   INTRODUCTION ..........................................  1

1.2   ASSEMBLY LANGUAGE .....................................  1

1.3   OPERATING ENVIRONMENT .................................  2

1.4   ASSEMBLER PROCESSING ..................................  2

CHAPTER 2 ........................................................  3

2.1   INTRODUCTION ..........................................  3

2.2   SOURCE STATEMENT FORMAT ...............................  3

2.2.1   Label Field ....................................  3

2.2.2   Operation Field ................................  4

2.2.3   Operand Field ..................................  4

2.2.3.1  M6800/6801 Operand Syntax ................  5

2.2.3.2  M6800/M68HC04 Operand Syntax .............  5

2.2.3.3  M6805/M68HC05 Operand Syntax .............  5

2.2.3.4  M6809 Operand Syntax .....................  5

2.2.3.5  M68HC11 Operand Syntax ...................  6

2.2.3.6  Expressions ..............................  6

2.2.3.7  Operators ................................  7

2.2.3.8  Symbols ..................................  7

2.2.3.9  Constants ................................  7

2.2.4   Comment Field ..................................  8

2.3   ASSEMBLER OUTPUT ......................................  9

CHAPTER 3 - RUNNING THE ASSEMBLERS ...............................  10

3.1   ASSEMBLER INVOCATION ..................................  10

3.2   ERROR MESSAGES ........................................  11

CHAPTER 4 - ASSEMBLER DIRECTIVES .................................  12

4.1   INTRODUCTION ..........................................  12

4.2   BSZ - BLOCK STORAGE OF ZEROS ..........................  12

4.3   EQU - EQUATE SYMBOL TO A VALUE ........................  13

4.4   FCB - FORM CONSTANT BYTE ..............................  13

4.5   FCC - FORM CONSTANT CHARACTER STRING ..................  13

4.6   FDB - FROM DOUBLE BYTE CONSTANT .......................  13

4.7   FILL - FILL MEMORY ....................................  14

4.8   OPT - ASSEMBLER OUTPUT OPTIONS ........................  14

4.9   ORG - SET PROGRAM COUNTER TO ORIGIN ...................  14

4.10   PAGE - TOP OF PAGE ...................................  15

4.11   RMB - RESERVE MEMORY BYTES ...........................  15

4.12   ZMB - ZERO MEMORY BYTES ..............................  15

APPENDIX A - CHARACTER SET .......................................  16

APPENDIX B - ADDRESSING MODES ....................................  18

B.1   M6800/M6801 ADDRESSING MODES ..........................  18

B.2   M6804/68HC04 ADDRESSING MODES .........................  19

B.3   M6805/68HC05 ADDRESSING MODES .........................  21

i

TABLE OF CONTENTS

B.4   M6809 ADDRESSING MODES ................................  22

B.5   M68HC11 ADDRESSING MODES ..............................  26

APPENDIX C - DIRECTIVE SUMMARY ...................................  28

APPENDIX D - ASSEMBLER LISTING FORMAT ............................  29

APPENDIX E - S-RECORD INFORMATION ................................  30

E.1   INTRODUCTION ..........................................  30

E.2   S-RECORD CONTENT ......................................  30

E.3   S-RECORD TYPES ........................................  30

E.4   S-RECORD EXAMPLE ......................................  31

REVISION 1.03 NEW FEATURES

3/29/90 - B. L. Olney

While using this assembler I found several things that I didn't like

about it.  Seems as though the source code was available I decided to

make a few changes so that it is easier for me to use.  The changes I

made are listed below.  So far the assembler should be 100% compatible

with older versions you may have.  These are only enhancements.  Where

applicable I have modified the text of this manual to document these

changes.  If you find any buggs with the new version please direct them

to me.  (801) 778-4410 or write: attn: Bruce L. Olney, IOMEGA Corp.,

1821 w. 4000 s. Bldg. 7, Roy, UT  84067.

- Version number is now displayed at the beginning of assembly.  Version

number at this writing is 1.03.

- The lost clusters bug is fixed.

- Comments may begin on any column if prefixed with a semicolon ';'.

- Hex constants may use a trailing 'H' or 'h.'

- Octal constants may use a trailing 'Q' or 'q.'

- Binary constants may use a trailing 'B' or 'b.'

- Shift operators are now supported. '{' is shift left and '}' is right.

They work the same as the 'C' operators '<<' and '>>'.  (Operand on

left is word to be shifted.  Operand on right is number of bits to

shift.)

- db is a synonymn for fcb.

- dw is a synonymn for fdb.

- ds is a synonymn for rmb.

- Formatting of symbol table enhanced.

- Number of errors now reported at end of assembly.

ii

CHAPTER 1

GENERAL INFORMATION

1.1   INTRODUCTION

This is the user's reference manual for the IBM-PC hosted Motorola

Freeware 8 bit cross assemblers.  It details the features and

capabilities of the cross assemblers, assembler syntax and directives,

options, and listings. It is intended as a detailed reference and an

introduction for those unfamiliar with Motorola assembler syntax and

format.  Those experienced with Motorola assembler products may wish

to examine the file ASEMBLER.DOC available with the cross assemblers,

which briefly describes the differences between these assemblers and

earlier, non-pc based versions.

Assemblers are programs that process assembly language source program

statements and translate them into executable machine language object

files.  A programmer writes his source program using any text editor

or word processor that can produce an ASCII text output.  With some

word processors this is known as "non document" mode.  Non document

mode produces a file without the non-printable embedded control

characters that are used in document formating.  (Caution: assembling

a file that has been formatted with embedded control characters may

produce assembler errors.  The solution is to convert the source file

to ASCII text.) Once the source code is written, the source file is

assembled by processing the file via the assembler.

Cross assemblers (such as the Motorola Freeware Assemblers) allow

source programs written and edited on one computer (the host) to

generate executable code for another computer (the target).  The

executable object file can then be downloaded and run on the target

system.  In this case the host is an IBM-PC or compatible and the

target system is based on a Motorola 8-bit microprocessor (6800, 6801,

6803, 6805, 68HC05, 6809, or 68HC11).

The assemblers are the executable programs AS*.EXE where * is any of

0, 1, 4, 5, 9, or 11 depending on which microprocessor you are writing

code for.  The details of executing the assembler programs are found

in Chapter 3.  The assembly language format and syntax for the various

processors is very similar with slight variations due to varied

programming resources (instructions, addressing modes, and registers).

These variations are explained in Appendix B.

1.2   ASSEMBLY LANGUAGE

The symbolic language used to code source programs to be processed by

the Assembler is called assembly language. The language is a

collection of mnemonic symbols representing: operations (i.e., machine

instruction mnemonics or directives to the assembler), symbolic names,

operators, and special symbols. The assembly language provides

mnemonic operation codes for all machine instructions in the

instruction set. The instructions are defined and explained in the

Programming Reference Manuals for the specific devices, available from

Motorola. The assembly language also contains mnemonic directives

Freeware Assemblers User's Manual

which specify auxiliary actions to be performed by the Assembler.

These directives are not always translated into machine language.

1.3   OPERATING ENVIRONMENT

These assemblers will run on any IBM-PC, XT, AT, PS-2, or true

compatible.  The assemblers may be run off of a floppy disk drive or

they may be copied onto a hard drive for execution.  DOS 2.0 or later

is required.

1.4   ASSEMBLER PROCESSING

The Macro Assembler is a two-pass assembler. During the first pass,

the source program is read to develop the symbol table. During the

second pass, the object file is created (assembled) with reference to

the table developed in pass one. It is during the second pass that

the source program listing is also produced.

Each source statement is processed completely before the next source

statement is read. As each statement is processed, the Assembler

examines the label, operation code, and operand fields. The operation

code table is scanned for a match with a known opcode.  During the

processing of a standard operation code mnemonic, the standard

machine code is inserted into the object file. If an Assembler

directive is being processed, the proper action is taken.

Any errors that are detected by the Assembler are displayed before the

actual line containing the error is printed. If no source listing is

being produced, error messages are still displayed to indicate that

the assembly process did not proceed normally.

2

Freeware Assemblers User's Manual

CHAPTER 2

CODING ASSEMBLY LANGUAGE PROGRAMS

2.1   INTRODUCTION

Programs written in assembly language consist of a sequence of source

statements. Each source statement consists of a sequence of ASCII

characters ending with a carriage return.  Appendix A contains a list

of the supported character set.

2.2   SOURCE  STATEMENT FORMAT

Each source statement may include up to four fields:  a label (or "*"

for a comment line), an operation (instruction mneumonic or assembler

directive), an operand, and a comment.

2.2.1   Label Field

The label field occurs as the first field of a source statement. The

label field can take one of the following forms:

1. An asterisk (*) as the first character in the label field indicates

that the rest of the source statement is a comment. Comments are

ignored by the Assembler, and are printed on the source listing only

for the programmer's information.

2. A whitespace character (blank or tab) as the first character

indicates that the label field is empty. The line has no label and is

not a comment.

3. A symbol character as the first character indicates that the line

has a label. Symbol characters are the upper or lower case letters a-

z,  digits 0-9, and the special characters, period (.), dollar sign

($), and underscore (_). Symbols consist of one to 15 characters, the

first of which must be alphabetic or the special characters period (.)

or underscore (_).  All characters are significant and upper and lower

case letters are distinct.

A symbol may occur only once in the label field. If a symbol does

occur more than once in a label field, then each reference to that

symbol will be flagged with an error.

With the exception of some directives, a label is assigned the value

of the program counter of the first byte of the instruction or data

being assembled. The value assigned to the label is absolute.

Labels may optionally be ended with a colon (:).  If the colon is

used it is not part of the label but merely acts to set the label off

from the rest of the source line.  Thus the following code fragments

are equivalent:

here:  deca

bne here

3

Freeware Assemblers User's Manual

here   deca

bne here

A label may appear on a line by itself.  The assembler interprets this

as set the value of the label equal to the current value of the

program counter.

The symbol table has room for at least 2000 symbols of length 8

characters or less.  Additional characters up to 15 are permissible at

the expense of decreasing the maximum number of symbols possible in

the table.

2.2.2   Operation Field

The operation field occurs after the label field, and must be preceded

by at least one whitespace character. The operation field must contain

a legal opcode mneumonic or an assembler directive.  Upper case

characters in this field are converted to lower case before being

checked as a legal mneumonic.  Thus 'nop', 'NOP', and 'NoP' are

recognized as the same mneumonic. Entries in the operation field may

be one of two types:

Opcode. These correspond directly to the machine instructions. The

operation code includes any register name associated with the

instruction.  These register names must not be separated from the

opcode with any whitespace characters.  Thus 'clra' means clear

accumulator A, but 'clr a' means clear memory location identified by

the label 'a'.

Directive. These are special operation codes known to the Assembler

which control the assembly process rather than being translated into

machine instructions.

2.2.3   Operand Field

The operand field's interpretation is dependent on the contents of the

operation field. The operand field, if required, must follow the

operation field, and must be preceded by at least one whitespace

character. The operand field may contain a symbol, an expression, or a

combination of symbols and expressions separated by commas.

The operand field of machine instructions is used to specify the

addressing mode of the instruction, as well as the operand of the

instruction.   The following tables summarize the operand field

formats for the various processor families. (NOTE:  in these tables

parenthesis "()" signify optional elements and angle brackets "<>"

denote an expression is inserted.  These syntax elements are present

only for clarification of the format and are not inserted as part of

the actual source program.  All other characters are significant and

must be used when required.)

4

Freeware Assemblers User's Manual

2.2.3.1  M6800/6801 Operand Syntax

The format of the operand field for M6800/6801 instructions is:

Operand Format           M6800/M6801 Addressing Mode

--------------           ---------------------------

no operand               accumulator and inherent

<expression>             direct, extended, or relative

#<expression>            immediate

<expression>,X           indexed

Details of the M6800/6801 addressing modes may be found in Appendix B.

2.2.3.2  M6804/68HC Operand Syntax

For the M6804/68HC04, the following operand formats exist:

Operand Format                M6804/68HC04 Addressing Mode

--------------                ----------------------------

no operand                    accumulator and inherent

<expression>                  direct, extended, or relative

#<expression>                 immediate

<expression>                  bit set or clear

<expression>,<expression>     bit test and branch

[<x> or <y>]                  register indirect

<expression>,#<expression>    move indirect

Details of the M6804/68HC04 addressing modes may be found in Appendix

B.

2.2.3.3  M6805/M68HC05 Operand Syntax

For the M6805/68HC05, the operand formats are:

Operand Format                M6805/68HC05 Addressing Mode

--------------                ----------------------------

no operand                    accumulator and inherent

<expression>                  direct, extended, or relative

#<expression>                 immediate

<expression>,X                indexed

<expression>,<expression>     bit set or clear

<expression>,<expression>,<expression>     bit test and branch

Details of the M6805/68HC05 addressing modes may be found in Appendix

B.

2.2.3.4  M6809 Operand Syntax

For the M6809, the following  operand formats are used:

5

Freeware Assemblers User's Manual

Operand Format           M6809 Addressing Mode

--------------           ---------------------

no operand               accumulator and inherent

<expression>             direct, extended, or relative

#<expression>            immediate

<expression>,X           indexed

<<expression>            forced direct

><expression>            forced extended

<expression>]            extended indirect

<expression>,R           indexed

<<expression>,R          forced 8-bit offset indexed

><expression>,R          forced 16-bit offset indexed

[<expression>,R]         indexed indirect

<[<expression>,R]        forced 8-bit offset indexed indirect

>[<expression>,R]        forced 16-bit offset indexed indirect

Q+                       auto increment by 1

Q++                      auto increment by 2

[Q++]                    auto increment indirect

-Q                       auto decrement by

--Q                      auto decrement by 2

[--Q]                    auto decrement indirect

W1,[W2,...,Wn]           immediate

where R is one of the registers PCR, S, U, X, or Y, and Q is one of

the registers S, U, X, or Y.   Wi  (i=1 to n) is one of the symbols A,

B, CC, D, DP, PC, S, U, X, or Y.

Details of the M6809 addressing modes may be found in Appendix B.

2.2.3.5  M68HC11 Operand Syntax

For the M68HC11, the following operand formats exist:

Operand Format                M68HC11 Addressing Mode

--------------                -----------------------

no operand                    accumulator and inherent

<expression>                  direct, extended, or relative

#<expression>                 immediate

<expression>,X                indexed with X register

<expression>,Y                indexed with Y register

<expression> <expression>     bit set or clear

<expression> <expression> <expression>     bit test and branch

The bit manipulation instruction operands are separated by spaces in

this case since the HC11 allows bit manipulation instructions on

indexed addresses.  Thus a ',X' or ',Y' may be added to the final two

formats above to form the indexed effective address calculation.

Details of the M68HC11 addressing modes may be found in Appendix B.

The operand fields of assembler directives are described in Chapter 4.

2.2.3.6  Expressions.  An expression is a combination of symbols,

constants, algebraic operators, and parentheses. The expression is

used to specify a value which is to be used as an operand.

6

Freeware Assemblers User's Manual

Expressions may consist of symbols, constants, or the character '*'

(denoting the current value of the program counter) joined together by

one of the operators: + - * / % & | ^ .

2.2.3.7  Operators.  The operators are the same as in c:

+    add

-    subtract

*    multiply

/    divide

%    remainder after division

&    bitwise and

|    bitwise or

^    bitwise exclusive or

Expressions are evaluated left to right and there is no provision for

parenthesized expressions.  Arithmetic is carried out in signed two-

complement integer precision (that's 16 bits on the IBM PC).

2.2.3.8  Symbols.  Each symbol is associated with a 16-bit integer

value which is used in place of the symbol during the expression

evaluation.  The asterisk (*) used in an expression as a symbol

represents the current value of the location counter (the first byte

of a multi-byte instruction).

2.2.3.9  Constants.  Constants represent quantities of data that do

not vary in value during the execution of a program.  Constants may be

presented to the assembler in one of five formats: decimal,

hexadecimal, binary, or octal, or ASCII.  The programmer indicates the

number format to the assembler with the following prefixes:

$    HEX            or trailing H

%    BINARY         or trailing B

@    OCTAL          or trailing Q

'    ASCII

Unprefixed constants are interpreted as decimal.  The assembler

converts all constants to binary machine code and are displayed in the

assembly listing as hex.

A decimal constant consists of a string of numeric digits. The value

of a decimal constant must fall in the range 0-65535, inclusive.  The

following example shows both valid and invalid decimal constants:

VALID     INVALID   REASON INVALID

-----     -------   --------------

12        123456    more than 5 digits

12345     12.3      invalid character

A hexadecimal constant consists of a maximum of four characters from

the set of digits (0-9) and the upper case alphabetic letters (A-F),

and is preceded by a dollar sign ($).  Hexadecimal constants must be

7

Freeware Assemblers User's Manual

in the range $0000 to $FFFF. The following example shows both valid

and invalid hexadecimal constants:

VALID     INVALID   REASON INVALID

-----     -------   --------------

$12       ABCD      no preceding "$"

$ABCD     $G2A      invalid character

$001F     $2F018    too many digits

A binary constant consists of a maximum of 16 ones or zeros preceded

by a percent sign (%).  The following example shows both valid and

invalid binary constants:

VALID     INVALID             REASON INVALID

-----     -------             --------------

%00101    1010101             missing percent

%1        %10011000101010111  too many digits

%10100    %210101             invalid digit

An octal constant consists of a maximum of six numeric digits,

excluding the digits 8 and 9, preceded by a commercial at-sign (@).

Octal constants must be in the ranges @0 to @177777.  The following

example shows both valid and invalid octal constan

ts:

VALID     INVALID   REASON INVALID

-----     -------   --------------

@17634    @2317234  too many digits

@377      @277272   out of range

@177600   @23914    invalid character

A single ASCII character can be used as a constant in expressions.

ASCII constants are preceded by a single quote ('). Any character,

including the single quote, can be used as a character constant.   The

following example shows both valid and inval

id character constants:

VALID     INVALID   REASON INVALID

-----     -------   --------------

'*        'VALID    too long

For the invalid case above the assembler will not indicate an error.

Rather it will assemble the first character and ignore the remainder.

2.2.4   Comment Field

The last field of an Assembler source statement is the comment field.

This field is optional and is only printed on the source listing for

documentation purposes. The comment field is separated from the

operand field (or from the operation field if no operand is required)

by at least one whitespace character. The comment field can contain

any printable ASCII characters.

Comments may also begin with a semicolon.  When a semicolon is used

the comment may begin in any column.

8

Freeware Assemblers User's Manual

2.3   ASSEMBLER OUTPUT

The Assembler output includes an optional listing of the source

program and an object file which is in the Motorola S Record format.

Details of the S Record format may be found in Appendix E.   The

Assembler will normally suppress the printing of the source listing.

This condition, as well as others, can be overridden via options

supplied on the command line that invoked the Assembler.

Each line of the listing contains a reference line number, the address

and bytes assembled, and the original source input line.  If an input

line causes more than 6 bytes to be output (e.g. a long FCC

directive), additional bytes (up to 64) are listed on succeeding lines

with no address preceding them.

The assembly listing may optionally contain a symbol table or a cross

reference table of all symbols appearing in the program. These are

always printed at the end of the assembly listing if either the symbol

table or cross reference table options (Paragraph 4.8) are in effect.

The symbol table contains the name of each symbol, along with its

defined value. The cross reference table additionally contains the

assembler-maintained source line number of every reference to every

symbol. The format of the cross reference table is shown in Appendix

D.

9

Freeware Assemblers User's Manual

CHAPTER 3

RUNNING THE ASSEMBLERS

3.1   ASSEMBLER INVOCATION

The Motorola Freeware Assembly programs are named as*.exe where '*' is

any of 0, 1, 4, 5, 9, or 11 depending on which processor family you

wish to assemble code for.  For example, to generate M6800 code run

the as0.exe program.  To generate M68HC05 code run the as5.exe

program, and so forth.  To run the assembler enter the following

command line:

as*   file1 (file2 . . . ) ( - option1 option2 . . . )

where file1, file2, etc are the names of the source files you wish to

assemble.  The source filenames may have extensions but the assembler

does not check for any particular extension ( however, do not use the

.S19 extension since that is the extension of the object file created

by the assembler.  Its creation would overwrite the source file when

it is written to the disk).

The options are one or more of the following:

l    enables output listing

no   disables output listing (default).

cre  enables the cross reference table generation

s    enables the symbol table generation

c    enables cycle counting

noc  disables cycle counting

The minus sign preceding the option should be separated from the last

file name by a space.  These options may also be indicated to the

assembler by the use of the OPT directive in the source file.  The OPT

directive is described in Paragraph 4.8.

The object file created is written to disk and given the name

'FILENAME.S19' where 'FILENAME' is the name of the first source file

specified on the command line.  Any errors and the optional listing

(if specified) are displayed on the screen.  The listing and/or error

messages may be saved to a file for later examination or printing by

append an i/o redirection command to the command line.  On the PC i/o

redirection is indicated with the greater-than ('>') symbol followed

by any new or existing file name.

Command line examples:

The command line

as5 myfile

would run the M6805/68HC05 assembler on the source file 'myfile'.  The

object file would be written to 'myfile.s19' and any errors would

appear on the screen.

10

Freeware Assemblers User's Manual

The command line

as9 test.asm nexttest.s -l

would run the M6809 assembler on the source files 'test.asm' and

'nexttest.s'.  The object file would be written to 'test.s19' and any

errors and the assembly listing would appear on the screen.

The command line

as9 test.asm nexttest.s -l cre s >test.lst

would run the M6809 assembler on the source files 'test.asm' and

'nexttest.s'.  The object file would be written to 'test.s19'.  A

listing would be created followed by a symbol table and cross

reference which would all be written to the file test.lst

.

3.2   ERROR MESSAGES

Error diagnostic messages are placed in the listing file just before

the line containing the error.  The format of the error line is:

Line_number:   Description of error

or

Line_number:   Warning ---- Description of error

Errors in pass one cause cancellation of pass two.  Warning do not

cause cancellation of pass two but are indications of a possible

problem.  Error messages are meant to be self-explanatory.

If more than one file is being assembled, the file name precedes the

error:

File_name,Line_number:  Description of error

Some errors are classed as fatal and cause an immediate termination of

the assembly.  Generally this happens when a temporary file cannot be

created or is lost during assembly.

11

Freeware Assemblers User's Manual

CHAPTER 4

ASSEMBLER DIRECTIVES

4.1   INTRODUCTION

The Assembler directives are instructions to the Assembler, rather

than instructions to be directly translated into object code. This

chapter describes the directives that are recognized by the Freeware

Assemblers. Detailed descriptions of each directive are arranged

alphabetically. The notations used in this chapter are:

( )  Parentheses denote an optional element.

XYZ  The names of the directives are printed in capital letters.

< >  The element names are printed in lower case and contained in

angle brackets.  All elements outside of the angle brackets '<>' must

be specified as-is. For example, the syntactical element (<number>,)

requires the comma to be specified if the optional element <number> is

selected. The following elements are used in the subsequent

descriptions:

<comment>      A statement's comment field

<label>        A statement label

<expression>   An Assembler expression

<expr>         An Assembler expression

<number>       A numeric constant

<string>       A string of ASCII characters

<delimiter>    A string delimiter

<option>       An Assembler option

<symbol>       An Assembler symbol

<sym>          An Assembler symbol

<sect>         A relocatable program section

<reg list>     M6809 register list

<reg exp>      M6809 register expression

In the following descriptions of the various directives, the syntax,

or format, of the directive is given first.  This will be followed

with the directive's description.

4.2   BSZ - BLOCK STORAGE OF ZEROS

(<label>) BSZ <expression> (<comment>)

The BSZ directive causes the Assembler to allocate a block of bytes.

Each byte is assigned the initial value of zero. The number of bytes

allocated is given by the expression in the operand field. If the

expression contains symbols that are either undefined or forward

referenced (i.e. the definition occurs later on in the file), or if

the expression has a value of zero, an error will be generated.

12

Freeware Assemblers User's Manual

4.3   EQU - EQUATE SYMBOL TO A VALUE

<label> EQU <expression> (<comment>)

The EQU directive assigns the value of the expression in the operand

field to the label. The EQU directive assigns a value other than the

program counter to the label. The label cannot be redefined anywhere

else in the program.  The expression cannot contain any forward

references or undefined symbols.  Equates with forward references are

flagged with Phasing Errors.

4.4   FCB - FORM CONSTANT BYTE or DB - DEFINE BYTE

(<label>) FCB <expr>(,<expr>,...,<expr>) (<comment>)

The FCB directive may have one or more operands separated by commas.

The value of each operand is truncated to eight bits, and is stored in

a single byte of the object program.  Multiple operands are stored in

successive bytes. The operand may be a numeric constant, a character

constant, a symbol, or an expression. If multiple operands are

present, one or more of them can be null (two adjacent commas), in

which case a single byte of zero will be assigned for that operand.

An error will occur if the upper eight bits of the evaluated operands'

values are not all ones or all zeros.

4.5   FCC - FORM CONSTANT CHARACTER STRING

(<label>) FCC <delimiter><string><delimiter> (<comment>)

The FCC  directive is used to store ASCII strings into consecutive

bytes of memory.  The byte storage begins at the current program

counter.  The label is assigned to the first byte in the string. Any

of the printable ASCII characters can be contained in the string.  The

string is specified  between two identical delimiters which can be any

printable ASCII character. The first non-blank character after the FCC

directive is used as the delimiter.

Example:

LABEL1    FCC  , ABC,

assembles ASCII ABC at location LABEL1

4.6 FDB - FORM DOUBLE BYTE CONSTANT or DW - DEFINE WORD

(<label>) FDB <expr>(,<expr>,...,<expr>) (<comment>)

The FDB directive may have one or more operands separated by commas.

The 16-bit value corresponding to each operand is stored into two

consecutive bytes of the object program.  The storage begins at the

current program counter.  The label is assigned to the first 16-bit

value. Multiple operands are stored in successive bytes.  The operand

may be a numeric constant, a character constant, a symbol, or an

expression. If multiple operands are present, one or more of them can

13

Freeware Assemblers User's Manual

be null (two adjacent commas), in which case two bytes of zeros will

be assigned for that operand.

4.7 FILL - FILL MEMORY

(<label>) FILL <expression>,<expression>

The FILL directive causes the assembler to initialize an area of

memory with a constant value.  The first expression signifies the one

byte value to be placed in the memory and the second expression

indicates the total number of successive bytes to be initialized.  The

first expression must evaluate to the range 0-255.  Expressions cannot

contain forward references or undefined symbols.

4.8 OPT - ASSEMBLER OUTPUT OPTIONS

OPT <option>(,<option>,...,<option>) (<comment>)

The OPT directive is used to control the format of the Assembler

output. The options are specified in the operand field, separated by

commas. All options have a default condition.  Some options can be

initialized from the command line that invoked the Assembler, however

the options contained in the source file take precedence over any

entered on the command line. In the following descriptions, the

parenthetical inserts specify "DEFAULT", if the option is the default

condition.  All options must be entered in lower case.

c -  Enable cycle counting in the listing.  The total cycle count

for that instruction will appear in the listing after the assembled

bytes and before the source code.

cre -  Print a cross reference table at the end of the source

listing. This option, if used, must be specified before the first

symbol in the source program is encountered.  The cross reference

listing format may be found in Appendix D.

l -  Print the listing from this point on.  A description of the

listing format can be found in Appendix D.

noc -  (DEFAULT) Disable cycle counting in the listing.  If the "c"

option was used previously in the program, this option will cause

cycle counting to cease until the next "OPT c" statement.

nol -   (DEFAULT)  Do not print the listing from this point on.  An

"OPT l" can re-enble listing at a later point in the program.

s -  Print symbol table at end of source listing.  The symbol table

format can be found in Appendix D.

4.9 ORG - SET PROGRAM COUNTER TO ORIGIN

ORG <expression> (<comment>)

The ORG directive changes the program counter to the value specified

by the expression in the operand field. Subsequent statements are

14

Freeware Assemblers User's Manual

assembled into memory locations starting with the new program counter

value. If no ORG directive is encountered in a source program, the

program counter is initialized to zero.  Expressions cannot contain

forward references or undefined symbols.

4.10 PAGE - TOP OF PAGE

PAGE

The PAGE directive causes the Assembler to advance the paper to the

top of the next page. If no source listing is being produced, the PAGE

directive will have no effect. The directive is not printed on the

source listing.

4.11 RMB - RESERVE MEMORY BYTES or DS - DEFINE(reserve) STORAGE

(<label>) RMB <expression> (<comment>)

The RMB directive causes the location counter to be advanced by the

value of the expression in the operand field. This directive reserves

a block of memory the length of which in bytes is equal to the value

of the expression. The block of memory reserved is not initialized to

any given value. The expression cannot contain any forward references

or undefined symbols.  This directive is commonly used to reserve a

scratchpad or table area for later use.

4.12 ZMB - ZERO MEMORY BYTES  (same as BSZ)

(<label>) ZMB <expression> (<comment>)

The ZMB directive causes the Assembler to allocate a block of bytes.

Each byte is assigned the initial value of zero. The number of bytes

allocated is given by the expression in the operand field. If the

expression contains symbols that are either undefined or forward

references, or if the expression has a value of zero, an error will be

generated.

15

Freeware Assemblers User's Manual

APPENDIX A

CHARACTER SET

The character set recognized by the Freeware Assemblers is a subset of

ASCII. The ASCII code is shown in the following figure. The following

characters are recognized by the Assembler:

1. The upper case letters A through Z and lower case letters a

through z.

2. The digits 0 through 9.

3. Five arithmetic operators:  +, -, *, / and % (remainder

after division).

4. Three logical operators:  &, |, and ^.

5. The special symbol characters:  underscore (_), period (.),

and dollar sign ($).  Only the underscore and period may be

used as the first character of a symbol.

6. The characters used as prefixes for constants and

addressing modes:

#    Immediate addressing

$    Hexadecimal constant

&    Decimal constant

@    Octal constant

%    Binary constant

'    ASCII character constant

7. The characters used as suffixes for constants and

addressing modes:

,X   Indexed addressing

,PCR M6809 indexed addressing

,S   M6809 indexed addressing

,U   M6809 indexed addressing

,Y   M6809 and M68HC11 indexed addressing

8. Three separator characters:  space, carriage return, and

comma.

9. The character "*" to indicate comments.  Comments may

contain any printable characters from the ASCII set.

10. The special symbol backslash "\" to indicate line

continuation.  When the assembler encounters the line

continuation character it fetches the next line and adds it

to the end of the first line.  This continues until a line

is seen which doesn't end with a backslash or until the

system maximum buffer size has been collected (typically

greater or equal to 256).

16

Freeware Assemblers User's Manual

11. For the M6809 Assembler, the character "<" preceding an

expression to indicate direct addressing mode or 8-bit

offset in indexed mode, and the character ">" preceding an

expression to indicate extended addressing mode or 16-bit

offset in indexed mode.

12. For the M6809 Assembler, the characters used to indicate

auto increment and auto decrement in the indexed mode:  +,

++, -, --.

ASCII CHARACTER CODES

BITS 4 to 6 -- 0    1    2    3    4    5    6    7

-----------    -------------------------------------

0    NUL  DLE  SP   0    @    P    `    p

B    1    SOH  DC1  :    1    A    Q    a    q

I    2    STX  DC2  !    2    B    R    b    r

T    3    ETX  DC3  #    3    C    S    c    s

S    4    EOT  DC4  $    4    D    T    d    t

5    ENQ  NAK  %    5    E    U    e    u

0    6    ACK  SYN  &    6    F    V    f    v

7    BEL  ETB  '    7    G    W    g    w

T    8    BS   CAN  (    8    H    X    h    x

O    9    HT   EM   )    9    I    Y    i    y

A    LF   SUB  *    :    J    Z    j    z

3    B    VT   ESC  +    ;    K    [    k    {

C    FF   FS   ,    <    L    \    l    ;

D    CR   GS   -    =    M    ]    m    }

E    SO   RS   .    >    N    ^    n    ~

F    S1   US   /    ?    O    _    o    DEL

17

Freeware Assemblers User's Manual

APPENDIX B

ADDRESSING MODES

B.1   M6800/M6801 ADDRESSING MODES.

INHERENT OR ACCUMULATOR ADDRESSING

The M6800 includes some instructions which require no operands. These

instructions are self-contained and employ the inherent addressing or

the accumulator addressing mode.

IMMEDIATE ADDRESSING

Immediate addressing refers to the use of one or two bytes of

information that immediately follow the operation code in memory.

Immediate addressing is indicated by preceding the operand field with

the pound sign or number sign character (#). The expression following

the # will be assigned one or two bytes of storage, depending on the

instruction.

RELATIVE ADDRESSING

Relative addressing is used by branch instructions. Branches can only

be executed within the range -126 to +129 bytes relative to the first

byte of the branch instruction.  For this mode, the programmer

specifies the branch address expression and places it in the operand

field. The actual branch offset is calculated by the assembler and put

into the second byte of the branch instruction.  The offset is the

two's complement of the difference between the location of the byte

immediately following the branch instruction and the location of the

destination of the branch.  Branches out of bounds are flagged as

errors by the assembler.

INDEXED ADDRESSING

Indexed addressing is relative to the index register. The address is

calculated at the time of instruction execution by adding a one-byte

displacement (in the second byte of the instruction) to the current

contents of the X register. Since no sign extension is performed on

this one-byte displacement, the offset cannot be negative. Indexed

addressing is indicated by the characters ",X" following the

expression in the operand field. The special case of ",X", without a

preceding expression, is treated as "0,X".

DIRECT AND EXTENDED ADDRESSING

Direct and extended addressing utilize one (direct) or two (extended)

bytes to contain the address of the operand. Direct addressing is

limited to the first 256 bytes of memory. Direct and extended

addressing are indicated by only having an expression in the operand

field. Direct addressing will be used by the Assembler whenever

possible.

18

Freeware Assemblers User's Manual

B.2   M6804/M68HC04 ADDRESSING MODES.

INHERENT OR ACCUMULATOR ADDRESSING

The M6800 includes some instructions which require no operands. These

instructions are self-contained and employ the inherent addressing or

the accumulator addressing mode.

IMMEDIATE ADDRESSING

Immediate addressing refers to the use of one byte of information that

immediately follows the operation code in memory. Immediate addressing

is indicated by preceding the operand field with the pound sign or

number sign character (#). The expression following the # will be

assigned one byte of storage.

RELATIVE ADDRESSING

Relative addressing is used by branch instructions. Branches can only

be executed within the range -15 to +16 bytes relative to the first

byte of the branch instruction.  For this mode, the programmer

specifies the branch address expression and places it in the operand

field. The actual branch offset is calculated by the assembler and put

into the second byte of the branch instruction.  The offset is the

two's complement of the difference between the location of the byte

immediately following the branch instruction and the location of the

destination of the branch.  Branches out of bounds are flagged as

errors by the assembler.

DIRECT AND EXTENDED ADDRESSING

Direct and extended addressing utilize byte to contain the address of

the operand. Direct addressing is limited to the first 256 bytes of

memory.  Extended addressing concatenates the four least-significant

bits of the opcode with the byte following the opcode to form a 12-bit

address. Direct and extended addressing are indicated by only having

an expression in the operand field.  Direct addressing will be used by

the Assembler whenever possible.

SHORT DIRECT

Some opcodes allow 4 memory locations in data space ram ($80, $81,

$82, and $83 to be referenced as part of the opcode.  The opcode

determines the data space RAM location, and the instruction is only

one byte.  The X and Y registers are at locations $80 and $81,

respectively.   An expression used with short direct addressing must

not be forward referenced (that is its definition must occur before,

not after this point in the file) and must equate to the range $80-

$83.

BIT SET AND CLEAR

In the bit set/clear addressing mode, the bit to be set or cleared is

part of the opcode.  The byte following the opcode specifies the

direct address of the byte which will have the bit set or cleared.

Any bit in the 256 byte data space memory that can be written (with

19

Freeware Assemblers User's Manual

the exception of the data direction registers) can be set or cleared

with these two byte instructions.

BIT TEST AND BRANCH

The bit test and branch addressing mode is a combination of the direct

addressing and relative addressing.  The bit to be tested, and it

condition (set or clear), is included in the opcode.  The data space

address of the byte to be tested is in the single byte immediately

following the opcode byte and follows direct addressing rules.  The

third byte is sign extended by the processor during execution to form

the 12-bit relative address which is added to the program counter if

the condition is true.  This allows branches based on any readable bit

in the data space.  The branch span is -125 to +130 from the opcode

address.  The branch target address is used by the programmer to

signify the relative offset -- the assembler calculates the offset

value.  Branches out of bounds are flagged as errors by the

assembler.

REGISTER INDIRECT

In the register indirect mode, the operand is at the address in data

space pointed to by the contents of one of the indirect registers, X

or Y.  The particular indirect register is encoded in bit 4 of the

opcode by the assembler.  The assembler  operand syntax for register

indirect is

[<X> or <Y>]

MOVE IMMEDIATE

The MVI (move immediate) instruction has its own format:

mvi   <expression 1>,#<expression 2>

where <expression 1> is a direct address and <expression 2> is the

data value to be written.

MISCELLANEOUS SYNTAX ISSUES

The registers in the 6804/HC6804 are memory locations and have

addresses assigned to them.  The assembler has predefined

a = A = $FF

b = B = $80

c = C = $81

This also means that for the '04 assembler clr x is equivalent to clrx

since x is both a register and a memory location.

The '04 series has separate program and data spaces.  There is no

program memory in the range $10-$7F.  Bytes assembled into that range

will go into the data space.

20

Freeware Assemblers User's Manual

B.3   M6805/68HC05 ADDRESSING MODES.

INHERENT OR ACCUMULATOR ADDRESSING

The M6805 includes some instructions which require no operands. These

instructions are self-contained, and employ the inherent addressing or

the accumulator addressing mode.

IMMEDIATE ADDRESSING

Immediate addressing refers to the use of one byte of information that

immediately follows the operation code in memory. Immediate addressing

is indicated by preceding the operand field with the pound sign or

number sign character (#). The expression following the # will be

assigned one byte of storage.

RELATIVE ADDRESSING

Relative addressing is used by branch instructions. Branches can only

be executed within the range -126 to +129 bytes relative to the first

byte of the branch instruction. For this mode, the programmer

specifies the branch address expression and places it in the operand

field. The actual branch offset is calculated by the assembler and put

into the second byte of the branch instruction.  The offset is the

two's complement of the difference between the location of the byte

immediately following the branch instruction and the location of the

destination of the branch.  Branches out of bounds are flagged as

errors by the assembler.

INDEXED ADDRESSING

Indexed addressing is relative to the index register. The address is

calculated at the time of instruction execution by adding a one- or

two-byte displacement to the current contents of the X register. The

displacement immediately follows the operation code in memory.  If the

displacement is zero, no offset is added to the index register.  In

this case, only the operation code resides in memory.  Since no sign

extension is performed on a one-byte displacement, the offset cannot

be negative. Indexed addressing is indicated by the characters ",X"

following the expression in the operand field.  The special case of

",X", without a preceding expression, is treated as "0,X". Some

instructions do not allow a two-byte displacement.

DIRECT AND EXTENDED ADDRESSING

Direct and extended addressing utilize one (direct) or two (extended)

bytes to contain the address of the operand. Direct addressing is

limited to the first 256 bytes of memory.  Direct and extended

addressing are indicated by only having an expression in the operand

field. Some instructions do not allow extended addressing.  Direct

addressing will be used by the Macro Assembler whenever possible.

BIT SET OR CLEAR

The addressing mode used for this type of instruction is direct,

although the format of the operand field is different from the direct

addressing mode described above. The operand takes the form

21

Freeware Assemblers User's Manual

<expression 1>, <expression 2>. <expression 1> indicates which bit is

to be set or cleared.  It must be an absolute expression in the range

0-7. It is used in generating the operation code.  <expression 2> is

handled as a direct address, as described above.  Since the bit

manipulation address is direct, only the first 256 locations may be

operated on by bit manipulation operations.

BIT TEST AND BRANCH

This combines two addressing modes: direct and relative. The format of

the operand is: <expression 1>, <expression 2>, <expression 3>.

<expression 1> and <expression 2> are handled in the same manner as

described above in "bit set or clear". <expression 3> is used to

generate a relative address, as described above in "relative

addressing".

B.4  M6809 ADDRESSING MODES.

INHERENT OR ACCUMULATOR ADDRESSING

The M6809 includes some instructions which require no operands. These

instructions are self-contained, and employ the inherent addressing or

the accumulator addressing mode.

IMMEDIATE ADDRESSING

Immediate addressing refers to the use of one or two bytes of

information that immediately follow the operation code in memory.

Immediate addressing is indicated by preceding the operand field with

the pound sign or number sign (#) -- i.e., #<expression>.  The

expression following the # will be assigned one or two bytes of

storage, depending on the instruction. All instructions referencing

the accumulator "A" or "B", or the condition code register "CC", will

generate a one-byte immediate value. Also, immediate addressing used

with the PSHS, PULS, PSHU, and PULU instructions generates a one-byte

immediate value. Immediate operands used in all other instructions

generate a two-byte value.

The register list operand does not take the form #<expression> but

still generates one byte of immediate data. The form of the operand

is:

R1,R2,...,Rn

where Ri (i=1 to n)  is one of the symbols A, B, CC, D, DP, PC, S, U,

X or Y. The number and type of symbols vary, depending on the specific

instruction.

For the instructions PSHS, PULS, PSHU, and PULU, any of the above

register names may be included in the register list. The only

restriction is that "U" cannot be specified with PSHU or PULU, and "S"

cannot be specified with PSHS  or PULS. The one-byte immediate value

assigned to the operand is calculated by the assembler and is

determined by the registers specified. Each register name causes the

assembler to set a bit in the immediate byte as follows:

22

Freeware Assemblers User's Manual

Register  Bit

--------  ---

PC      7

U,S     6

Y       5

X       4

DP      3

B,D     2

A,D     1

CC      0

For the instructions EXG and TFR, exactly two of the above register

names must be included in the register list. The other restriction is

the size of the registers specified. For the EXG instruction, the two

registers must be the same size. For the TFR instruction, the two

registers must be the same size, or the first can be a 16-bit register

and the second an 8-bit register. In the case where the transfer is

from a 16-bit register to an 8-bit register, the least significant 8

bits are transferred.  The 8-bit registers are A, B, CC, and DP.  The

16-bit registers are D, PC, S, U, X, and Y.  The one-byte immediate

value assigned to the operand by the assembler is determined by the

register names. The most significant four bits of the immediate byte

contain the value of the first register name; the least significant

four bits contain the value of the second register, as shown by the

following table:.

Register  Value (hex)

--------  -----------

D          0

X          1

Y          2

U          3

S          4

PC         5

A          8

B          9

CC         A

DP         B

RELATIVE  ADDRESSING

Relative addressing is used by branch instructions. There are two

forms of the branch instruction. The short branch can only be executed

within the range -126 to +129 bytes relative to the first byte of the

branch instruction. For this mode, the programmer specifies the branch

address expression and places it in the operand field. The actual

branch offset is calculated by the assembler and put into the second

byte of the branch instruction. The long branch can execute in the

full range of addressing from 0000-FFFF (hexadecimal) because a two-

byte offset is calculated by the assembler and put into the operand

field of the branch instruction.  The offset is the two's complement

of the difference between the location of the byte immediately

23

Freeware Assemblers User's Manual

following the branch instruction and the location of the destination

of the branch.

DIRECT AND EXTENDED ADDRESSING

Direct and extended addressing utilize one (direct) or two (extended)

bytes to contain the address of the operand. Direct and extended

addressing are indicated by having only an expression in the operand

field (i.e., <expression>). Direct addressing will be used whenever

possible.

Regardless of the criteria described above, it is possible to force

the Assembler to use the direct addressing mode by preceding the

operand with the "<" character. Similarly, extended addressing can be

forced by preceding the operand with the ">" character.  These two

operand forms are: <<expression> and ><expression>.

INDEXED ADDRESSING

Indexed addressing is relative to one of the index registers. The

general form is <expression>,R. The address is calculated at the time

of instruction execution by adding the value of <expression> to the

current contents of the index register. The other general form is

[<expression>,R].   In this indirect form, the address is calculated

at the time of instruction execution by first adding the value of

<expression> to the current contents of the index register, and then

retrieving the two bytes from the calculated address and address+1.

This two-byte value is used as the effective address of the operand.

The allowable forms of indexed addressing are described below.  In the

description below, R refers to one of the index registers S, U, X, or

Y.

The accumulator offset mode allows one of the accumulators to be

specified instead of an <expression>. Valid forms are:.

<acc>,R and [<acc>,R]

where <acc>  is one of the accumulators A, B, or D. This form

generates a one-byte operand (post-byte only). When accumulator A or B

is specified, sign extension occurs prior to adding the value in the

accumulator to the index register.

The valid forms for the automatic increment/decrement mode are shown

below.  For each row, the three entries shown are equivalent.

R+       ,R+      0,R+

-R       ,-R      0,-R

R++      ,R++     0,R++

--R      ,--R     0,--R

[R++]    ,R++]    [0,R++]

[--R]    [,--R]   [0,--R]

In this form, the only valid expression is 0. Like the accumulator

offset mode, this form generates a one-byte operand (post-byte only).

24

Freeware Assemblers User's Manual

The valid forms for the expression offset mode are:

R       ,R     <expression>,R

[R]     [,R]   [<expression>,R]

<R      <,R    <<expression>,R

<[R]    <[,R]  <[<expression>,R]

>R      >,R    ><expression>,R

>[R]    >[,R]  >[<expression>,R]

The "<" and ">" characters force an 8- or 16-bit offset, respectively,

and are described below. If no expression is specified, or if an

expression with a value of zero is specified, only the postbyte of

the operand is generated. If an expression with a value in the range

-16 to +15 is specified without indirection, a one- byte operand is

generated which contains the expression's value, as well as the index

register indicator. At execution time, the expression's value is

expanded to 16 bits with sign extension before being added to the

index register.

All other forms will generate a post-byte, as well as either a one- or

two-byte offset which contains the value of the expression. The size

of the offset is determined by the type and size of the expression.

Expressions with values in the range -128 to +127 generate an 8-bit

offset.   All other cases will result in a 16-bit offset being

generated. In the case where an 8-bit offset is generated, the value

is expanded to 16 bits with sign extension at execution time.

Regardless of the criteria described above, it is possible to force

the Assembler to generate an 8-bit offset by preceding the operand

with the "<" character. Similarly, a 16-bit offset can be forced by

preceding the operand with the ">" character.

If the relative address calculated is not in the range -128 to +127,

or if the expression references a symbol that has not yet been

defined, a two-byte offset is generated after the post-byte. A one-

byte offset is generated if the relative address is in the range -128

to +127.

Like the expression offset mode, a one-byte offset can be forced by

preceding the operand with a "<".  A ">" forces a two-byte offset. A

byte overflow error is generated if a one-byte offset is forced when

the relative address is not in the range -12

8 to +127.

The extended indirect mode has the form:

[<expression>]

Although extended indirect is a logical extension of the extended

addressing mode, this mode is implemented using an encoding of the

postbyte under the indexed addressing mode.  A post-byte and a two-

byte offset which contains the value of the expression is generated.

25

Freeware Assemblers User's Manual

B.5  M68HC11 ADDRESSING MODES.

PREBYTE

The number of combinations of instructions and addressing modes for

the 68HC11 is larger than that possible to be encoded in an 8-bit word

(256 combinations).  To expand the opcode map, certain opcodes ($18,

$1A, and $CD) cause the processor to fetch the next address to find

the actual instruction.  These opcodes are known as prebytes and are

inserted automatically by the assembler for those instructions that

require it.l  In general the instructions contained in the alternate

maps are those involving the Y register or addressing modes that

involve the Y index register.  Thus the programmer make the tradeoff

between the convenience of using the second index register and the

additional time and code space used by the prebyte.

INHERENT OR ACCUMULATOR ADDRESSING

The M68HC11 includes some instructions which require no operands.

These instructions are self-contained, and employ the inherent

addressing or the accumulator addressing mode.

IMMEDIATE ADDRESSING

Immediate addressing refers to the use of one or more bytes of

information that immediately follow the operation code in memory.

Immediate addressing is indicated by preceding the operand field with

the pound sign or number sign character (#). The expression following

the # will be assigned one byte of storage.

RELATIVE ADDRESSING

Relative addressing is used by branch instructions. Branches can only

be executed within the range -126 to +129 bytes relative to the first

byte of the branch instruction. For this mode, the programmer

specifies the branch address expression and places it in the operand

field. The actual branch offset is calculated by the assembler and put

into the second byte of the branch instruction.  The offset is the

two's complement of the difference between the location of the byte

immediately following the branch instruction and the location of the

destination of the branch.  Branches out of bounds are flagged as

errors by the assembler.

INDEXED ADDRESSING

Indexed addressing is relative one of the index registers X or Y.  The

address is calculated at the time of instruction execution by adding a

one-byte displacement to the current contents of the X register. The

displacement immediately follows the operation code in memory.  If the

displacement is zero, zero resides in the byte following the opcode.

Since no sign extension is performed on a one-byte displacement, the

offset cannot be negative. Indexed addressing is indicated by the

characters ",X" following the expression in the operand field.  The

special case of ",X", without a preceding expression, is treated as

"0,X".

26

Freeware Assemblers User's Manual

DIRECT AND EXTENDED ADDRESSING

Direct and extended addressing utilize one (direct) or two (extended)

bytes to contain the address of the operand. Direct addressing is

limited to the first 256 bytes of memory.  Direct and extended

addressing are indicated by only having an expression in the operand

field.  Direct addressing will be used by the Assembler whenever

possible.

BIT(S) SET OR CLEAR

The addressing mode used for this type of instruction is direct,

although the format of the operand field is different from the direct

addressing mode described above. The operand takes the form

<expression 1> <expression 2> where the two expressions are separated

by a blank.   <expression 1> signifies the operand address and may be

either a direct or an indexed address.  When the address mode is

indexed, <expression  1> is followed by ',R' where R is either X or Y.

This allows bit manipulation instructions to operate across the

complete 64K address map.  <expression 2> is the mask byte.  The

bit(s) to be set or cleared are indicated by ones in the corresponding

location(s) in the mask byte.  The mask byte must be an expression in

the range 0-255 and is encoded by the programmer.

BIT TEST AND BRANCH

This combines two addressing modes: direct or indexed and relative.

The format of the operand is: <expression 1> <expression 2>

<expression 3> where the expressions are separated by blanks.

<expression 1>  identifies the operand an may indicate either a direct

or indexed address.  Indexed addresses are signified with ',R'

following the expression where R is either X or Y. <expression 2> is

the mask byte.  The bit(s) to be set or cleared are indicated by ones

in the corresponding location(s) in the mask byte.  The mask byte must

be an expression in the range 0-255 and is encoded by the programmer.

<expression 3> is used to generate a relative address, as described

above in "relative addressing".

27

Freeware Assemblers User's Manual

APPENDIX C

DIRECTIVE SUMMARY

A complete description of all directives appears in Chapter 4.

ASSEMBLY CONTROL

ORG  Origin program counter

SYMBOL DEFINITION

EQU  Assign permanent value

DATA DEFINITION/STORAGE ALLOCATION

BSZ  Block storage of zero; single bytes

FCB  Form constant byte

FCC  Form constant character string

FDB  Form constant double byte

FILL Initialize a block of memory to a constant

RMB  Reserve memory; single bytes

ZMB  Zero Memory Bytes; same and BSZ

LISTING CONTROL

OPT c     Enable cycle counting

OPT cre   Print cross reference table

OPT l     Print source listing from this point

OPT nol   Inhibit printing of source listing from this point

OPT s     Print symbol table

PAGE Print subsequent statements on top of next page

28

Freeware Assemblers User's Manual

APPENDIX D

ASSEMBLER LISTING FORMAT

The Assembler listing has the following format:

LINE#   ADDR  OBJECT CODE BYTES       [ # CYCLES]  SOURCE LINE

The LINE# is a 4 digit decimal number printed as a reference.  This

reference number is used in the cross reference.  The ADDR is the hex

value of the address for the first byte of the object code for this

instruction.   The OBJECT CODE BYTES are the assembled object code of

the source line in hex.  If an source line causes more than 6 bytes

to be output (e.g. a long FCC directive), additional bytes (up to 64)

are listed on succeeding lines with no address preceding them.

The # CYCLES will only appear in the listing if the "c" option is in

effect.  It is enclosed in brackets which helps distinguish it from

the source listing.  The SOURCE LINE is reprinted exactly from the

source program, including labels.

The symbol table has the following format:

SYMBOL    ADDR

The symbol is taken directly from the label field in the source

program.  The ADDR is the hexadecimal address of the location

referenced by the symbol.

The cross reference listing has the following format:

SYMBOL   ADDR   *LOC1 LOC2 LOC3 ...

The SYMBOL and ADDR are the same as above. The * indicates the start

of the line reference numbers.  The LOCs are the decimal line numbers

of the assembler listing where the label occurs.

29

Freeware Assemblers User's Manual

APPENDIX E

S-RECORD INFORMATION

E.1   INTRODUCTION

The S-record output format encodes program and data object modules

into a printable (ASCII) format.  This allows viewing of the object

file with standard tools and allows display of the module while

transferring from one computer to the next or during loads between a

host and target.  The S-record format also includes information for

use in error checking to insure the integrity of data transfers.

E.2   S-RECORD CONTENT

S-Records are character strings made of several fields which identify

the record type, record length, memory address, code/data, and

checksum.  Each byte of binary data is encoded as a 2-character

hexadecimal number:  the first character representing the high-order

4 bits, and the second the low-order 4 bits of the byte.

The 5 fields which comprise an S-record are:

TYPE  RECORD LENGTH   ADDRESS   CODE/DATA  CHECKSUM

The fields are defined as follows:

FIELD      CHARACTERS                   CONTENTS

-----      ----------                   --------

Type           2          S-record type - S1, S9, etc.

Record         2          The count of the character pairs in the

length                    record, excluding the type and record

length.

Address      4, 6,        The 2-, 3-, or 4-byte address at which

or 8         the data field is to be loaded into

memory.

Code/data    0-2n         From 0 to n bytes of executable code,

memory loadable data, or descriptive

information.

Checksum       2          The least significant byte of the one's

complement of the sum of the values

represented by the pairs of characters

making up the record length, address,

and the code/data fields.

Each record may be terminated with a CR/LF/NULL.

E.3   S-RECORD TYPES

Eight types of s-records have been defined to accommodate various

30

Freeware Assemblers User's Manual

encoding, transportation, and decoding needs.  The Freeware

assemblers use only two types, the S1 and S9:

S1   A record containing code/data and the 2-byte

address at which the code/data is to reside.

S9   A termination record for a block of S1 records. The address

field may optionally contain the 2-byte address of the

instruction to which control is to be passed.  If not

specified, the first entry point specifica

tion encountered in the object module input will be used.

There is no code/data field.

E.4   S-RECORD EXAMPLE

The following is a typical S-record module:

S1130000285F245F2212226A000424290008237C2A

S11300100002000800082629001853812341001813

S113002041E900084E42234300182342000824A952

S107003000144ED492

S9030000FC

The module consists of four code/data records and an S9 termination

record.

The first S1 code/data record is explained as follows:

S1   S-record type S1, indicating a code/data record to be

loaded/verified at a 2-byte address.

13   Hex 13 (decimal 19), indicating 19 character pairs,

representing 19 bytes of  binary data, follow.

00   Four-character 2-byte address field:  hex address 0000,

indicates location where the following data is to be loaded.

The next 16 character pairs are the ASCII bytes of the actual

program code/data

2A   Checksum of the first S1 record.

The second and third S1 code/data records each also contain $13

character pairs and are ended with checksums.  The fourth S1 code/data

record contains 7 character pairs.

The S9 termination record is explained as follows:

S9   S-record type S9, indicating a termination record.

03   Hex 03, indicating three character pairs (3 bytes) to

follow.

00   Four character 2-byte address field, zeroes.

00

FC   Checksum of  S9 record.

31