PROGRAM CONVERSION

============================================================================

This distribution contains two library files and headers, psu120_lib and

mx12p_lib, for driving the PSU120 and MX12P.  They are based (and follow

quite closely) the libraries from Adimec.  This section describes the

changes made to convert to Linux.  The goal has been to maintain source

compatibility with the Adimec libraries, so that any program written

for the DLLs needs only to be re-compiled and linked under Unix.

psu120_lib

==========

1.  The following type substitutions have been used:

HANDLE    -> int

LPSTR     -> uchar*

BYTE      -> unsigned char  ("uchar")

BOOL      -> int

_DLL_     -> null string

WINAPI    -> null string

HINSTANCE -> null string

DWORD     -> null string

LPVOID    -> null string

FALSE     -> 0

TRUE      -> 1

Of these, only replacing HANDLE by int is required (by the definition of

a file descriptor under Unix).

2.  TransmitCmd(int, TCmd*) has replaced TransmittCmd(int, TCmd*).  This is

an internal function and should not be called by any other programs.

3.  Polling has not been tested.

4.  A new error code ERR_INT_COMM, which is returned when there are problems

communicating with the PSU120 (flushing the serial buffer, no ACK), is

used in RemoteExecuteCmd.

5.  We call usleep at then end of TransmitCmd and ReceiveByte rather than

setting up a delay via termio.

6.  The ReceivePSUMessage code from Adimec has two timeout checks built

into it that this library does not.  The program could hang (or loop

endlessly) if no data arrives.

7.  The error codes ERR_INT conflict with ERR_PSU.  Since these are internal,

we shift them to -21 to -24.

mx12p_lib

=========

NOTE THE NEW FUNCTION PrepContextTable()!  The user context structure

used in the library is not automatically initialized --- you need to

do it yourself!

1.  New public function:

void PrepContextTable(void)

Initializes the user context structure used in this library.  You need

to call it at the start of your program (in the Adimec library, this

occurs automatically when the DLL is loaded).

int CamAttention(int)

Wrapper for PSUATTENTION.  The function checks that the port number is

valid and then sends an ATT command to the PSU.  A non-negative return

value means that the PSU120 is still responding to commands.

int CamVersion(int, char*)

Gets the version string from the PSU120 and writes it into the character

array.  The array must be large enough to hold the string

(PSU_DATA_SIZE + 1).

2.  New private function to library:

int findQIntegrationTime(TCamParameters*)

Code calculating the integration time given sensor type, pixel clock,

read-out speed, read-out mode, and interlacing.

3.  CamClose will now return ERR_CAM_IHND if the given comm port isn't in

use.

4.  The polling functions have not been tested.

6.  Sensor and pixel clock values are written back into the IO registers

(not sure if this is necessary; the Adimec code doesn't do it).

7.  Derived parameters are recalculated after reading defaults from camera

to ensure the consistancy of the data.

8.  The header file for mx12p_lib has been split in two: the public file to

use to compile to the library (mx12p_lib.h) and an internal file with

all declarations for working with the PSU120 data (mx12p_internal.h)

[this second file had been put in the middle of the C code in the

Adimec version].

USING THE LIBRARIES

============================================================================

1.  Compile the libraries:

gcc $(CFLAGS) -c -o mx12p_lib.o mx12p_lib.c

gcc $(CFLAGS) -c -o psu120_lib.o psu120_lib.c

2.  Compile your program that will use these libraries.

3.  Link all together